Compute all the unnormalized associated Legendre functions.

Usage

-----

p = PLegendreA (lmax, z, [csphase])

Returns

-------

p : float, dimension ((lmax+1)*(lmax+2)/2)

An array of unnormalized associated Legendre functions up to degree lmax.

The index corresponds to l*(l+1)/2+m.

Parameters

----------

lmax : integer

The maximum degree of the associated Legendre functions to be computed.

z : float

The argument of the associated Legendre functions.

csphase : optional, integer, default = 1

If 1 (default), the Condon-Shortley phase will be excluded. If -1, the

Condon-Shortley phase of (-1)^m will be appended to the associated Legendre

functions.

Description

-----------

PLegendreA will calculate all of the unnormalized associated Legendre functions

up to degree lmax for a given argument. These are calculated using a standard

three-term recursion formula and hence will overflow for moderate values of l

and m. The index of the array corresponding to a given degree l and angular

order m corresponds to l*(l+1)/2+m. The integral of the associated Legendre

functions over the interval [-1, 1] is 2*(l+m)!/(l-m)!/(2l+1). The default is to

exclude the Condon-Shortley phase, but this can be modified by setting the

optional argument csphase to -1.